# LAB 3 – Introduction to SDR with ADALM-PLUTO

Course: Wireless Communications
Duration: ~2 hours
Platform: MATLAB + ADALM-PLUTO SDR

## Objective

The ADALM-PLUTO is a compact but powerful software-defined radio (SDR) that enables real-time experimentation with wireless signals.
In this lab, you will learn how to configure the Pluto SDR, transmit and receive complex baseband signals, capture IQ samples for offline processing, and detect practical receiver issues such as buffer overflows.

By the end of this lab, you will be able to:

- Initialize and configure one or more ADALM-PLUTO devices

- Run Pluto in loopback, single-host, or two-host configurations

- Transmit a continuous-wave (CW) complex baseband signal

- Capture single and multiple frames of IQ samples

- Detect and visualize overflow events

- Prepare captured IQ data for later spectrum and signal analysis

## Equipment & Requirements

- MATLAB R2024b (or newer recommended)

- Communications Toolbox Support Package for ADALM-PLUTO

- One or two ADALM-PLUTO SDRs

- USB cables

## Safety & Regulatory Notes

- Use unlicensed bands or wired/near-field operation

- Follow institutional and FCC/ITU regulations

# Part 0 – Setting Up the Pluto

### Hardware Setup

Before starting this or any other Pluto labs:

- Obtain one or two ADALM-PLUTO devices

- Follow the MATLAB setup instructions for the Pluto support package

- Plug in the Pluto device(s)

- Run the command once per device:

findPlutoRadio

If successful, the READY LED should be solid and LED1 should blink.

# Part 1 – Configuration Modes

This lab supports three configurations:

### 1) One-Device Loopback Mode

- Single Pluto

- TX and RX on the same device

- Simplest mode (use first)

loopback = true;

runTx = true;

runRx = true;

### 2) Two Devices / Single Host

- Two Plutos connected to one computer

- One TX, one RX

loopback = false;

runTx = true;

runRx = true;

### 3) Two Devices / Two Hosts

- One Pluto per computer

- Separate TX and RX scripts

% On both hosts

loopback = false;

% TX host

runTx = true;

runRx = false;

% RX host

runTx = false;

runRx = true;

Important: Choose only one configuration. Do not enable multiple configurations at the same time. For this lab, begin with Two Devices / Single Host Mode.

### TODO

Set the parameters below based on your chosen configuration:

loopback = false;

runTx = true;

runRx = true;

# Part 2 – Creating TX and RX Objects

Run the section that creates the Pluto TX and RX objects using the helper function plutoCreateTxRx.

Key parameters:

- Sample Rate: 30.72 MHz

- Samples per Frame: (2^{12})

Frames are blocks of samples processed together, which is standard in SDR systems.

# Part 3 – Generating a Continuous Wave (CW) Signal

We transmit a complex exponential, also called a continuous wave (CW) as shown in Lab script: This represents a single spectral tone in baseband.

### TODO

Plot:

- Real part of (x[n])

- Imaginary part of (x[n])

Label the time axis in microseconds (µs).

### Concept Check

- Why does a complex exponential correspond to a single frequency?

- What does the real part represent physically?

# Part 4 – Transmitting the Signal

The Pluto repeatedly transmits the frame using:

tx.transmitRepeat(x);

This continuously loops the signal, similar to a tone generator. If RX is disabled, stop here.

# Part 5 – Capturing One Frame of Data

We now capture one frame of received samples:

- Pluto ADC outputs integer values

- The Pluto ADC outputs signed integers; scaling normalizes the samples to approximately [-1,1]. Thus, we scale to floating-point for analysis

### TODO

Plot:

- Real part of the received signal

- Imaginary part of the received signal
vs. time in microseconds (µs)

### Concept Check

- Why are RX samples complex?

- Why is scaling required after ADC capture?

# Part 6 – Capturing Consecutive Frames & Overflow Detection

We now capture multiple frames in a loop.

Each RX call returns:

- r: received data

- valid: data validity flag

- over: overflow indicator

Overflow typically occur when MATLAB cannot read frames fast enough due to processing delays or plotting overhead.

### TODO

After running the loop:

- Print the frame numbers where overflow occurred

- If none occurred, print “No overflows detected”

### Concept Check

- What causes buffer overflows in SDR systems?

- Why do overflows break signal continuity?

# Part 7 – Concatenating Frames

Create a single vector containing all samples:

dataConcat = ...

### TODO

- Create dataConcat

- Plot real(dataConcat) vs time in milliseconds

If overflows occurred, you should see discontinuities.

### Concept Check

- Why do overflows cause discontinuities?

- How might real receivers prevent overflow?