LAB 3 – MANUAL

Objective:

In this lab, you will gain hands-on experience with software-defined radio (SDR) using the ADALM-PLUTO. You will learn how to configure a real RF receiver in MATLAB, visualize real-time spectra, understand practical receiver impairments (DC spur, noise floor), and perform basic spectrum sensing tasks such as frequency sweeps, waterfall plots, and channel power estimation.

Learning Outcomes:

By the end of this lab, you should be able to:

- Verify and configure an SDR  receiver in MATLAB

- Interpret real-time power spectral density (PSD) plots

- Understand the relationship between sample rate, bandwidth, and frequency resolution

- Identify practical RF artifacts (DC spur, noise floor)

- Perform frequency sweeps and create waterfall plots

- Estimate channel power and capture raw IQ data for offline analysis

Safety & Regulatory Notes:

- Receive-only lab – do NOT transmit.

- Use public broadcasts.

- Follow all institutional and FCC/ITU regulations.

Requirements

- MATLAB R2024b

- Communications Toolbox Support Package for ADALM-PLUTO

- DSP System Toolbox

- ADALM-PLUTO SDR (USB-connected)

- Note: If hardware is unavailable, the script automatically falls back to a synthetic signal so the workflow can still be explored.

Procedure

Step 1 – Launch MATLAB

Open MATLAB R2024b

Step 2 – Load the Lab Script

Open LAB3.m.

Part 0 – Setup & Device Check

Run Section: 0) Setup & Device Check

This section:

- Clears the workspace

- Sets SDR parameters: Sample rate (fs), center frequency (fc), receiver gain

- Attempts to detect the Pluto device.

Questions:

- What does the Pluto sample rate represent physically?

- Why is the sample rate equal to the observable bandwidth?

- What happens if the receiver gain is too high or too low?

Part 1 – Receiver & Spectrum Analyzer Setup

Run Section: 1) Create Receiver & Spectrum Analyzer

This section:

- Creates an sdrrx (‘Pluto’) receiver object

- Configures a Spectrum Analyzer object to display PSD

Observations

- Spectrum is shown in dBW/Hz

- Averaging reduces noise variance

Questions

- Why is the spectrum centered around 0 Hz in baseband?

- What does ‘power spectral density' mean physically?

- How does averaging affect the displayed spectrum?

Part 2 – Live Spectrum: Tuning & Gain Experiments

Run Section: 2) Live Spectrum – Tune, Gain, & Sample Rate Experiments

You will observe the real-time spectrum while:

- Changing center frequency

- Adjusting gain

- Modifying sample rate

Experiments

Try the following:

- Increase gain until clipping is visible

- Reduce gain until the signal disappears

- Change fs and observe:

  - Noise floor

  - Frequency resolution

Questions:

- What happens to the noise floor when gain is increased?

- How does increasing sample rate affect frequency resolution?

- Why does changing fc shift the observed spectrum?

Part 3 – DC Spur Mitigation

Run Section: 3) DC Spur Mitigation (Center Spike)

Many direct-conversion radios exhibit a DC spike at 0 Hz. This section:

- Displays spectrum before DC removal

- Removes DC by subtracting the mean

- Displays spectrum after DC mitigation

Questions

- What causes the DC spur in direct-conversion receivers?

- Why does subtracting mean reduce the DC spike?

- Why might more advanced DC blockers be required in practice?

Part 4 – Frequency Sweep & Waterfall Plot

Run Section: 4) Waterfall & Frequency Sweep

This section:

- Sweeps center frequency across the FM band

- Computes Welch PSD at each step

- Builds a waterfall plot (frequency vs sweep index)

Questions:

- What does a vertical ‘stripe’ in the waterfall indicate?

- Why is a waterfall useful for spectrum sensing?

- How does this relate to cognitive radio concepts?

Part 5 – Channel Power Estimation

Run Section: 5) Channel Power & Occupancy

This section:

- Finds the strongest spectral peak

- Estimates power in a BW window

- Reports power in dBW

Questions

- Why is channel power computed over a bandwidth, not a single frequency?

- How does this relate to regulatory spectral masks?

- What assumptions are made when estimating power from PSD?

Part 6 – IQ Capture for Offline Analysis

Run Section: 6) Record IQ to File

This section:

- Captures raw complex IQ samples

- Saves data to pluto_capture.mat

These IQ samples can later be used for:

- Demodulation

- Filtering

- Spectral analysis

- Machine learning experiments

Questions:

- Why are IQ samples complex-valued?

- What information is preserved in IQ data that magnitude-only spectra lose?

- How could this data be used in later labs?

Part 7 – Clean Up

Run Section: 7) Clean Up

- Releases hardware resources

- Safely closes the receiver